HANG ‘N FIND

Fred’s Creative Woodworking

www.FrederickCHill.com

Congratulations! You are now the proud owner of the Hang ‘N Find, produced by Fred’s Creative Woodworking.  The Hang ‘N Find will enable you to hang those rulers, triangles, squares, etc. that you use in drawing, etc. in your quilting studio. Go online to view the Hang ‘N Find and to see these instructions. http://www.frederickchill.com/sales_hangnfind.php

ASSEMBLY

I have included L-hooks that have threads that are a bit longer than the thickness of the Hang ‘N Find. This allows the 6 hooks to help stabilize the Hang ‘N Find once it is mounted on the wall. When you are ready to mount it, you will need to screw the 6 metal L-shaped brass hooks into the wood.  This will be done in two parts. The first part is simply to get them started in the wood so once you have the unit mounted on the wall you can easily finish the job. The second part occurs once you have the unit mounted and you screw them in so the screw flange is flush with the wood. You should be able to do most of this by hand, however, if you find it necessary, use a pair of pliers to hold the hooks while you screw them into the 6 predrilled small, evenly spaced holes. Probably the last quarter turn you will need the pliers to finish the job. Be careful to not over tighten the hook.  When the flange on the top of the screw thread comes in contact with the wood you should turn only about another 1/3 more in order to get the hook in its proper position for hanging your items (e.g., the short leg of the “L” of the hook needs to point up when you mount the hanger on the wall). This finishing part will assure that the ends of the screws protrude into the wall slightly and help provide support for the unit.

When you have all six hooks in place, you are ready to mount the Hang ‘N Find on a wall.

MOUNTING

Be certain that you find a suitable spot on the wall.  Keep in mind what you plan on hanging from the hooks. If you have long items, you will need to mount the holder higher on the wall. If they are really wide, you will need to allow for more space to the left or right of the hanger.

I have the drilled two mounting holes at normal wall stud distance (16” centers). Ideally you can find a stud in your wall and the two screws you use to mount the holder will hold into your studs.  Some hints on finding your wall studs: The drywall is generally either screwed or nailed to the wall. Often the heads of the screws or nails leave a tell-tale dimple that will let you know where the stud is. You can also rap on the wall with something like the handle of a screwdriver or hammer handle. You will hear the different pitch of the sound as you tap on the stud verses over the stud cavity.

If you can’t find the studs or if they are not spaced on 16” centers, you will need to go to the hardware store and pick up two drywall anchors.

Mount the Hang ‘N Find so it is horizontal. There are several ways to do this. Probably the easiest and often best way is to hold the Hang ‘N Find on the wall and have someone look at it while you level it. After all, leveling something is for visual effect so your eye can usually do a perfect job. Mark a straight line where you want the hanger to go. You can also use a carpenter’s level.  Draw a line under the level to show you where to place the Hang ‘N Find.  A third way is to measure from the floor or the ceiling to the height that you want the hanger to go. Place a pencil mark on the wall at the same height about 2 feet apart and connect your two points with a pencil line. This method is only as accurate as the level of the floor or ceiling so be careful if you use it.

Place your Hang ‘N Find against the level line on the wall and mark through the two screw holes with a pencil. I suggest that you run your screws into the wall at this point to see if there is a piece of hidden wood in the wall at this point. If there is, you are in luck as you can use it to hang the unit without resorting to using drywall anchors.  If you have to use the anchors, tap them into the wall to start and finish by carefully screwing the anchor into place. You can then easily run your screws through the two screw holes in the Hang ‘N Find and into the anchors and tighten it to the wall.  If you are lucky enough to have found two studs that align with the two predrilled holes in the hanger, simply attach the Hang ‘N Find.

Now finish screwing the L-hooks into their final position as indicated in Assembly above.

When the Hang ‘N Find is attached to the wall, push the two wood buttons into the screw holes to hide them. Tap them in place with either the handle of a hammer or a piece of wood as this will keep you from marking them. You can use a small dab of glue (most will do) to hold the buttons in place but this probably isn’t necessary.

Enjoy!  If you have problems hanging your Hang ‘N Find, call or e-mail me and I’ll help you long distance.

Hang ‘N Find contents:

One wooden hanger

Six L-shaped hooks

2 wood buttons

2 wood screws- #2 Phillips head